*********************************************

*                                           *

*    ASPI4OS2.SYS                           *

*    OS/2 SCSI Manager for the AHA-1x4x     *

*    Functional Overview                    *

*    Tom Shea   3/27/89                     *

*    OS/2 Version 1.1                       *

*                                           *

*********************************************

Introduction:

ASPI4OS2.SYS is an installable device driver providing

a generic multitasking interface to the AHA-154x line of

host adapters.  Any SCSI device driver can ATTACH to this

SCSI Manager to pass requests to the SCSI bus through the

Adaptec SCSI programming interface.  For more

information on the interface between the SCSI drivers and

the SCSI manager refer to the Adaptec OS/2 SCSI Programming

Interface Specification.

In order to support booting and ROM BIOS supported SCSI drives,

the SCSI manager will often be included in the DISK01

base driver provided in an OEM release of OS/2.  The

installable version is necessary when there is no base SCSI

support.  If there is base support for SCSI, loading the

installable version will result in the de-installation of

the base version and the re-routing of all SCSI requests to

the Installable SCSI manager.  As long as the base version

is sufficient to get the system booted, updates and

improvements to the SCSI manager can be incorporated into an

installable version without affecting the OEM's OS/2 release.

Implementation:

To load the SCSI manager, the following line should appear

in the config.sys BEFORE any SCSI device drivers:

DEVICE = [path] ASPI4OS2.SYS [Options]

PATH refers to the location in the filesytem of the SCSIMGR.SYS

file.

OPTIONS refers to a combination of command line options.

The possibilities are listed below:

/D - Display informational messages during initialization.

This flag is useful when there is some sort of problem

getting the driver to load as expected.

/L - Support SCSI LUNs other than zero.  This flag informs

the SCSI manager that LUNs other than zero should be

supported.