STANDARD MICROSYSTEMS CORPORATION

Microsoft Windows NT v3.1 NDIS v3.0 Driver

For TokenCard EliteSeries LAN Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1993 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes:

o  How to use EZStart to copy the Microsoft Windows NT version 3.1

NDIS spec 3.0 driver from the SuperDisk to your PC.

o  How to install the driver.

Note: This driver supports all SMC TokenCard EliteSeries

ISA and Micro Channel Token-Ring adapters.

COPYING THE DRIVER FILE

From the EZStart Driver Installation screen, select 'Copy Files'

to copy all the files used by this driver from the SuperDisk to

the Destination Directory recommended by EZStart, or to a different

directory that you specify.

The EZStart default Destination Directory is C:\. Please make

a backup copy of your SuperDisk before EZStart copies files to the

SuperDisk to create an installation diskette.

Note:  If you change the Destination Directory, make sure that the

DISK.ID file is in the root directory of the disk that you

are using, and that the OEMSETUP.INF and SMC8100.LAN files

reside in the root directory or a subdirectory on the disk

you are using.

INSTALLING THE DRIVER

1. Use EZStart to test and configure your Token-Ring adapter. Make

sure you write down your Token-Ring adapter IRQ, I/O Base, and

RAM Base settings.

2. Boot your computer from Windows NT.

3. From the Program Manager's "Main" window, double-click on the

"Control Panel" icon.

4. Double-click on "Network". The Network Settings dialog box

appears.

5. Select "Add Adapter".

6. Select "<Other> Requires disk from manufacturer".

7. Select "Continue".

8. When prompted, insert the SuperDisk into your computer's

floppy disk drive.

9. Type the Destination Directory where EZStart copied the

driver files (for example, C:\WIN_NT). If you are not

sure where the Destination Directory resides, look for

the SMC8100N.SYS file. This file was one of the files

copied by EZStart to the Destination Directory. When you

find the directory where this file resides, you will have

found the Destination Directory.

10. Select "OK".

11. When the next dialog box appears, select the SMC Token-Ring

adapter followed by "OK".

12. Choose the IRQ level, memory base address, and I/O port

address. Refer to the information you wrote down in step 1.

13. When the next two dialog boxes appear, select "OK" for

each one.

14. Shutdown and reboot your computer.

In the unlikely event you encounter any device driver errors

after Windows NT boots, check the event log in the

Administrative Tools icon to see whether the error is related

to your network adapter.

SMC and Standard Microsystems are registered trademarks; and

TokenCard, TokenCard Elite, TokenCard Elite/A, Elite Series,

EliteView, EZStart, and SuperDisk are trademarks of Standard

Microsystems Corporation.  Other products and company names are

registered trademarks or trademarks of their respective holders.