Proposed Specification for MIXER API

copyright Media Vision (c) 1991

version 0.6b -- Ken Nicholson

MCIMIXER.DRV	MCI interface to the mixer driver

MMMIXER.DLL  	Multimedia Mixer DLL

MVMIXER.DRV	Media Vision Mixer driver

THE MULTIMEDIA MIXER DLL

The Multimedia Windows Mixer DLL, as proposed by Media Vision, is a

library of generalized routines that provides a common,	device-independent

interface to software controllable mixers.  Its design is based on the uses

and functionality of real world audio mixers.

This DLL gives the MultiMedia application programmer control over a complete

range of audio capabilites.  Every conceivable audio mixing, patching,

equalization and amplification need can be handled by this DLL and its API.

It is capable of supporting mixing features far beyond those available

with today's PC hardware.

WHY A MULTIMEDIA MIXER ?

The Multimedia PC Specification version 1 (May 13,1991) calls for

"On-board analog audio mixing capabilities," requiring "... input

from three (recommended four) sources and [must] present the sources as

a stereo, line-level audio signal at the back panel. ... Individual

audio source and master digital volume control registers and extra

line-level audio sources are highly recommended."

Such hardware requires a set of standard functions calls (API's) that

will handle volume changes in a device-independent and extensible way.

Furthermore, there are a number of issues, apart from the setting of

an input's or an output's volume levels, that should be handled in a

standard way.  Functions such as equalization, special effects, patch

changes, device association, connection mapping, smooth timed transitions,

power-up settings and device sharing were not addressed by the November Beta

release of the Multimedia Windows Development Kit.

What is provided? The WAVEOUT and MIDIOUT drivers have Get- and SetVolume

entry points for control of a device's output volume.  In addition, the

Multimedia Extensions define an AUX device type that allows

applications to get and set the volumes of additional devices.  The

only two types of AUX devices defined are CD audio and auxillary

input.

In the current design of the Multimedia Extensions, there is no link

between an AUX device and the audio device associated with it.  It

may be assumed, when there is only one AUX device, that it controls

the audio output of the CD-ROM drive.  But in the presence of multiple

AUX devices there is no mechanism for an application to determine

which AUX device to control to change the CD volume.  Varying

AUX-to-Device association will cause chaos for application writers.

This raises the question of volume control for devices in general.

Most consumer audio devices (cassette decks, turntables, video disk

players, televisions) don't have a variable line output. These

devices rely on a mixer or integrated amplifier to control the volume

level.  The term "attenuator" is applied to controls that vary the

line output of a device.

Attenuation of PCM and MIDI audio output is not something users need

concern themselves with and therefore volume control functions do not

belong in waveout and midiout drivers; volume control is properly a

mixing function.  Users and multimedia authors will want to individually

adjust the relative volumes of a number of device outputs and this is the

primary need for a mixer.  The Mixer API solves this problem by supporting

volume controls on each of any number of mixer inputs as well supporting

volume controls for each mixer output.

Some professional audio mixers feature a myriad of knobs to enhance or

alter input audio.  Among these are controls for bass, midrange and treble,

controls to add reverb, effects and stereo-mono crossover.  In fact, Media

Vision's Pro-Audio Spectrum supports many of these features.  The

existing Multimedia Windows API's don't address these features.

Another feature of the MMMIXER.DLL API set is its ability to maintain and

coordinate mixer patch information.  A mixer device that supports

software selectable input patching would allow any one of several audio

devices to be "patched" to a mixer input.  Similarly an application

can interrogate the mixer API to find out which devices are connected

to which mixer inputs.

Fading the volume up of one sound source while fading out the volume

of another seems such an intrinsically necessary function for multimedia

that we have incorporated it into the DLL.  Without this feature, the

application programmer would be required to send auxSetVolume

messages in "ping-pong" fashion to the devices to be cross-faded.

Some users will have powered speakers with no volume control. MMWindows

start-up sounds are capable of producing DEAFENING DECIBELS OF DIN.

Mixer drivers conforming to this spec read WIN.INI to determine the

desired start-up, or mixer reset, volume settings.

Frequently an application will fill the entire screen and on occasion

a Multimedia application will suprise the user with unexpectedly loud

audio.  Such moments find the user frantically trying to bring up the

volume control application in order to lower the volume.  An equally

dismaying situation is starting a multimedia application only to

discover that the volumes are too low for the sound to be audible.

The MMMIXER.DLL intercepts the CTRL-ALT-U and CTRL-ALT-D sequences

and translates them into driver volume up and down calls.  This

allows the user to quickly change volumes up and down.

Media Vision has developed the Multimedia Windows DLL to address these and

other issues in audio control on the MPC.  We gratefully acknowledge

Microsoft and especially the members of the Multimedia Windows

Development Team for their advice and assistance in developing the

Multimedia Mixer DLL and the Media Vision Mixer driver.

MEDIA VISION IMPLEMENTATION

The Multimedia Mixer DLL is capable of supporting virtually any type

of mixing hardware.  The limitations of the Pro AudioSpectrum mixing

hardware should not be confused with the capabilities of the

Multimedia Mixer DLL.

MVMIXER.DRV is implemented as a single Mixer device.  This mixer has

seven input lines and two output lines.  The Pro AudioSpectrum 16 has

new mixer circuitry that allows 8 input lines.

The standard input lines are by default patched to:

1) FM SYNTHESIZER	      5) MICROPHONE

2) RECORD MONITOR    	      6) WAVE (PCM)

3) AUXILLARY INPUT	      7) PC speaker

4) INTERNAL CD		      8) Snd Blaster WAVE (mono PCM)

On the Pro AudioSpectrum and Pro AudioSpectrum Plus, Input #2 is the mix

of all devices being routed to Output #2.  For the Pro AudioSpectrum 16,

Input #2 becomes a second internal CD connector.

Each input can be connected to one (but not both) of the two outputs.

1) PLAY	  (Line Out)

2) RECORD (Wave In)

Output #1, apart from supporting volume control, supports treble and

bass, loudness, stereo enhance and mute.

Output #2  supports volume control on the Pro-16 only.

Input #2 should not be patched to Output #2.

Input #6 should be patched to Output #2 during playback for proper

waveform filtering.  This is only necessary on the 8-bit Pro

AudioSpectrum.

mixGetDevCaps returns the following structure:

MIXERCAPS MVMixer= {

MM_MEDIAVISION,             // manufacturer id

MM_PROAUD_MIXER,		// product id

(VERSION) 0x100,  		// version of the driver

"Pro AudioSpectrum",	// product name (NULL terminated string)

7,				// number of inputs  supported.

2,				// number of outputs supported.

9, 				// number of input patches supported.

3,				// number of output patches supported.

MIXERCAP_MANUALPATCHSWITCH,	// supports some manual   patching

NULL			// reserved

};

A Line Capabaility structure for the Pro Audio Spectrum Mixer

would look like this:

{

wNumber	    =0;   	   	// This is the current caps for input # 0

dwDeviceType=MIX_MICROPHONE	// Microphone currently patched;

+MIX_USER_CONNECTED;// Requires user to plug it in (for prompting)

wNumSoftPatches=1;	   	// This input always patched to the Microphone

wPatchNumber=0;	   		// the only mic

wNumChannels=2;	   		// The mic is actually mono with a splitter

dwSupport   =MIX_SUPPORT_LRVOLUME	// supported functionality

szIOname    ="Line 1: Microphone Jack"	// Input Name

szPname     ="MIC";		  	// patch name

}

Each Mixer has a built in number of INPUT and OUTPUT patches. They

are referenced by an ordinal number starting from zero.  Patch number

zero is always defined as NO CONNECT.

Here is the capability structure for Output #1:

{

wPatchNumber =1;	      	  // This patch's ordinal number

dwDeviceType =MIX_AMPLIFIER|	  // device connected is amplifier

MIX_LISTENER|	  // all output patches are listeners

MIX_USER_CONNECTED; // requires a cable connection

dwLineNumbers=1<<0; 		  // bit-field, lines connectable to

szPname[]    ="MASTER";		  // patch name (NULL terminated string)

dwAssociation=NULL;   		  // HIWORD=type, LOWORD=device

dwReserved1  =NULL;  	  	  // reserved for future use

}

//////////////////////////////////////////////////////////////////////

//////////////////////////////////////////////////////////////////////

MIXER API OVERVIEW

MIXER GENERAL FUNCTIONS:

mixGetNumDevs 		- returns the number of mixer devices

mixGetDevCaps 		- provides information on mixer device capability

mixOpen 		- opens mixer device

mixClose		- closes mixer device

mixGetErrorText 	- gets a text string corresponding to error number

MIXER PRESET FUNCTIONS:

mixReset		- resets mixer inputs, volumes, patches etc.

mixGetState 		- gets current state of the mixer device

mixSetState		- sets mixer state with optional timed fade

mixGetFadeStatus	- returns time remaining of current fadeprocess

// INPUT TO OUTPUT CONNECTION

mixGetConnections	- gets the connection map of an input or output

mixSetConnections	- sets input to output connection map

// LINE CONTROLS

mixGetControl		- gets the setting of a line's control

mixSetControl		- sets the level of a line's control

mixGetLineInfo		- gets functionality and current status of a line

// PATCH CALLS

mixGetPatch		- gets a line's patch number

mixSetPatch		- sets a line's patch information

mixGetPatchInfo		- returns information on a standard mixerpatch

// DEVICE ORIENTED CALLS

mixGetDeviceName	- converts device type to device name

mixGetDeviceLines	- finds which lines have a specified device

mixSetDeviceConnections	- performs abstract device connection

mixGetDeviceConnections - determines device types connected

MIXER DEVICE GENERAL FUNCTIONS

mixGetNumDevs 		- returns the number of mixer devices

mixGetDevCaps 		- provides information on mixer device capability

mixOpen 		- opens mixer device

mixClose		- closes mixer device

mixGetErrorText 	- gets a text string corresponding to error number

______________________________________________________________________

Syntax		WORD mixGetNumDevs(void)

This function retrieves the number of mixer devices

present in the system.

Parameters	None

Return Value	Returns the number of mixer devices present in the

system

______________________________________________________________________

Syntax		mixGetDevCaps(wDeviceID, lpCaps, wSize)

Parameters	WORD  wDeviceID

Identifies the mixer device to be queried.

LPMIXERCAPS  lpCaps

Specifies a far pointer to a MIXERCAPS structure.

This structure is filled with information about the

capabilities of the device.

WORD  wSize

Specifies the size of the MIXERCAPS structure.

Return Value	When the wDeviceID is non-zero, the return value will

be MIXERR_NOERR (zero) if the function was successful.

If wDeviceID is zero, the return value is the size of

the drivers MIXERCAPS structure.

Comments	Use mixGetDevCaps to determine the number of mixer

devices present in the system.  The Device ID specified

by wDeviceID varies from zero to one less than the number

of devices present. Only wSize bytes(or less of information

will be copied to the location pointed to by lpCaps.  If

wSize is 0, nothing will be copied and MMSYSERR_NOERROR is

returned.

See Also	mixGetNumDevs

______________________________________________________________________

Syntax		mixOpen(lphMixer, wDeviceID, dwFlags)

This function opens a specified mixer device.

Parameters	LPHMIXER lphMixer

Specifies a far pointer to an HMIXER handle.  This

location is filled with a handle identifying the

opened mixer device.

WORD  wDeviceID

DWORD dwFlags

Specifies flags for opening the device

Return Value	Returns zero if the function was successful.

Otherwise, it returns an error.

Comments	Use mixOpen before making any control/enquiry calls

to the mixer driver.

See Also	mixClose

______________________________________________________________________

Syntax		mixClose(hMixer)

Parameters	HMIXER	hMixer

Return Value	Returns zero if the function was successful.

Otherwise, it returns an error. Possible errors are:

MMSYSERR_INVALHANDLE

Specified device handle is invalid.

See Also	mixOpen

______________________________________________________________________

Syntax		mixGetErrorText(wError, lpText, wSize)

This function retrieves a textual description of the

error identified by the specified error number.

Parameters	WORD  wError

Specifies the error number.

LPSTR lpText

Specifies a far pointer to a buffer which is filled

with the textual error description.

WORD  wSize

Specifies the length of the buffer pointerd to by

lpText.

Return Value	Returns the length of the string copied to zero if the function was successful.

______________________________________________________________________

MIXER PRESET FUNCTIONS

mixReset		- resets mixer inputs, volumes, patches etc.

mixGetState 		- gets current state of the mixer device

mixSetState		- sets current state of the mixer device

mixGetFadeStatus	- returns time remaining of current fadeprocess

______________________________________________________________________

Syntax		mixReset(hMixer)

Resets the mixer to a default state.  The default

state is read from the WIN.INI file.

Parameters	HMIXER	hMixer

Identifies the mixer device to be reset.

Return Value	Returns zero if the function was successful.

Otherwise, it returns an error.

See Also	mixOpen, mixGetSetup, mixSetSetup

______________________________________________________________________

Syntax		mixGetState(hMixer, lphMixerState, lpwSize)

Returns a handle to a structure containing the

current state of the mixer.  The structure is

defined by the device.

Parameters	HMIXER	 hMixer

Identifies the mixer device to be used.

LPHANDLE lphMixerState

Specifies a far pointer to a handle to where the mixer

state information is saved.

LPWORD   lpwSize

Specifies a far pointer to a word where the size of

the mixer state information is stored.

Return Value	Returns zero if the function was successful.

Otherwise, it returns an error. Possible errors are:

MIXERR_BADMIXERPTR	null pointer to mixer

Comments	This function is used to save the current state of

the specified mixer device.

See Also	mixSetState

______________________________________________________________________

Syntax		mixSetState(hMixer,lpMixerState,wSize,dwTime,

dwFlags,dwCallback);

Restores the mixer to a saved state.

Parameters	HMIXER	hMixer

Identifies the mixer device to be used.

LPMIXERSTATE lpMixerState

Handle to a mixer state structure as returned by

the mixGetState function.

WORD	wSize

Size of the mixer state structure.

DWORD	dwTime

The high word is delay time in tenths of seconds.

The low word is the duration of the fade in tenths

of seconds.  A dwTime value of zero results in an

instant mixer setting.

DWORD	dwFlags

MIX_FADE_OVERRIDE	override a fade in progress

DWORD	dwCallback

Address of a procedure to be called when fade is

complete.  Note: this has not yet been implemented

Return Value	Returns zero if the function was successful.

Otherwise, it returns an error. Possible errors are:

MIXERR_FADEINPROGRESS

Comments	If wSize is not correct, this function will be

rejected.

See Also	mixGetState, mixGetFadeStatus

______________________________________________________________________

Syntax		mixGetFadeStatus(hMixer,lpdwTime)

Parameters	HMIXER	 hMixer

Identifies the mixer device to be used.

LPDWORD  lpdwTime

The high word is the delay time remaining in tenths

of seconds.

The low word is the fade time remaining in tenths

of seconds.

An lpdwTime value of zero indicates no fade is in

progress.

Return Value	Returns zero if the function was successful.

Comments	Status indicators displaying the time remaining of

a fade may wish to call this function upon receipt of

any of the following MIXER MESSAGES:

WM_MIX_CONTROLCHANGED

WM_MIX_CONNECTIONCHANGED

WM_MIX_PATCHCHANGED

See Also 	mixSetState

______________________________________________________________________

// INPUT TO OUTPUT CONNECTION

mixGetConnections	- gets the connection map of an input or output

mixSetConnections	- sets input to output connection map

______________________________________________________________________

Syntax		mixGetConnections(hMixer,WORD wLine, LPDWORD lpdwConnections);

Parameters	HMIXER	 hMixer

Identifies the mixer device to be used.

WORD 	 wLine

The low byte indicates the mixer line to get the

connections information for.

The high byte indicates whether the line is an

input line or an output line. The following

macros (equates) are used for the high byte:

MIX_INPUT

MIX_OUTPUT

LPDWORD  lpdwConnections

A far pointer to a DWORD where the connection

information is to be stored.  Each bit of the

double word represents a mixer line.  If the

bit is 1, wLine is connected to that line.

For example, assume wLine=0x0000, indicating Input

Line #1.  If that Input line is connected to Output

Line #0, bit 0 of *lpdwConnections will be set.

Logically, connection information can only be

maintained for a mixer with a maximum of 32 inputs

and 32 outputs.

Return Value	Returns zero if the function was successful.

Possible Errors:

MIXERR_INVALINPUT 	   illegal input line

MIXERR_INVALOUTPUT	   illegal output line

Comments

See Also 	mixSetConnections

______________________________________________________________________

Syntax		mixSetConnections(Mixer,WORD wLine, DWORD dwConnections);

Parameters	HMIXER	 hMixer

Identifies the mixer device to be used.

'

WORD 	 wLine

The low byte indicates the mixer line to get the

connections information for.

The high byte indicates whether the line is an

input line or an output line. The following

macros (equates) are used for the high byte:

MIX_INPUT

MIX_OUTPUT

DWORD  dwConnections

This parameter specifies the connection map for the

given line of the mixer.  If wLine refers to an

input Line, the outputs specified by this parameter

will be connected to that input. Bit 0 refers to

line 0, bit 1 to line 1, etc.

Return Value	Returns zero if the function was successful.

Possible Errors:

MIXERR_INVALINPUT 	   illegal input line

MIXERR_INVALOUTPUT	   illegal output line

Comments	This function allows inputs to be selectively patched

to one or more outputs.  It also allows outputs to

be connected to one or more inputs.  This function

allows the caller to say "connect input lines 1, 3 and 5

to output #1" or "connect input line 4 to outputs 1 and 2"

If the hardware cannot support all connections requested,

the mixer driver will connect the lower numbered lines

first.  Calling mixGetConnections after	mixSetConnections

is recommended for verification of requested connections.

See Also 	mixGetConnections

______________________________________________________________________

// LINE CONTROLS

mixGetControl		- gets the setting of a line's control

mixSetControl		- sets the level of a line's control

mixGetLineInfo		- gets support functionality of a line

______________________________________________________________________

Syntax		mixGetControl(hMixer, wLineNum, dwControl, lpdwSetting);

Parameters	HMIXER	hMixer

Identifies the mixer device to be used.

WORD  	wLineNum

The low byte indicates the mixer line to get the

connections information for.

The high byte indicates whether the line is an

input line or an output line. The following

macros (equates) are used for the high byte:

MIX_INPUT

MIX_OUTPUT

DWORD	dwControl

Specifies the control to get the setting of.

Here is the current list of possible controls:

MIX_SUPPORT_LRVOLUME       left-right volume control

MIX_SUPPORT_ALC	       	 Auto Level Control

MIX_SUPPORT_BMT	       	 B-M-T equalization

MIX_SUPPORT_CROSSOVER      crossover change

MIX_SUPPORT_LOUDNESS       loudness equalization

MIX_SUPPORT_MUTE	       	 channel mute

MIX_SUPPORT_REVERB	 reverb

MIX_SUPPORT_STEREOENHANCE  stereo enhance

MIX_SUPPORT_CUSTOM1	 custom effect #1

MIX_SUPPORT_CUSTOM2	 custom effect #2

MIX_SUPPORT_CUSTOM3	 custom effect #3

LPDWORD lpdwSetting

Specifies a far pointer to a location that will be

filled with the current Control setting.  For

stereo controls, the high-order word of this

location contains the left channel setting and

the low-order word contains the right channel setting.

A value of 0xFFFF represents full intensity and a value

of 0x0000 is full cutout.

Return Value	Returns zero if the function was successful.

Otherwise, it returns an error. Possible errors are:

MIXERR_INVALINPUT 	   illegal input line

MIXERR_INVALOUTPUT	   illegal output line

MIXERR_NOTSUPPORTED	        control not supported

See Also	mixSetControl

______________________________________________________________________

Syntax		mixSetControl(hMixer, wLineNum, dwControl, dwSetting)

Parameters	HMIXER	hMixer

Identifies the mixer device to be used.

WORD	wLineNum

The low byte indicates the mixer line to get the

connections information for.

The high byte indicates whether the line is an

input line or an output line. The following

macros (equates) are used for the high byte:

MIX_INPUT

MIX_OUTPUT

Specifies the input to set Control for

DWORD	dwControl

Specifies the control to set

Here is the current list of possible controls:

MIX_SUPPORT_LRVOLUME       left-right volume control

MIX_SUPPORT_ALC	       Auto Level Control

MIX_SUPPORT_BMT	       B-M-T equalization

MIX_SUPPORT_CROSSOVER      crossover change

MIX_SUPPORT_LOUDNESS       loudness equalization

MIX_SUPPORT_MUTE	       channel mute

MIX_SUPPORT_REVERB	       reverb

MIX_SUPPORT_STEREOENHANCE  stereo enhance

MIX_SUPPORT_CUSTOM1	       custom effect #1

MIX_SUPPORT_CUSTOM2	       custom effect #2

MIX_SUPPORT_CUSTOM3	       custom effect #3

DWORD	dwSetting

Specifies a far pointer to a location that will be

filled with the current Control setting.  For

stereo controls, the high-order word of this

location contains the left channel setting and

the low-order word contains the right channel setting.

A value of 0xFFFF represents full intensity and a value

of 0x0000 is full cutout.

Return Value	Returns zero if the function was successful.

Otherwise, it returns an error. Possible errors are:

MIXERR_INVALINPUT 	   illegal input line

MIXERR_INVALOUTPUT	   illegal output line

MIXERR_NOTSUPPORTED	   control not supported

See Also	mixGetControl, CONTROL SETTING NOTES

_____________________________________________________________________

Syntax		mixGetLineInfo(hMixer, wLineNum, lpInfo, wSize);

Retrieves information about the specified input's

capabilities.

Parameters	HMIXER	hMixer

Identifies the mixer device to be used.

WORD  	wLineNum

The low byte indicates the mixer line to get the

connections information for.

The high byte indicates whether the line is an

input line or an output line. The following

macros (equates) are used for the high byte:

MIX_INPUT

MIX_OUTPUT

LPMIXERLINEINFO  lpInfo

Specifiies a far pointer to be filled with the capability

information for the specified input.

WORD  wSize

Specifies the size of the LPMIXERLINEINFO structure.

Return Value	Returns zero if the function was successful.

Comments

See Also

______________________________________________________________________

// PATCH CALLS

mixGetPatch		- gets a line's patch number

mixSetPatch		- sets a line's patch information

mixGetPatchInfo		- returns information on a standard mixerpatch

______________________________________________________________________

Syntax		mixGetPatch(hMixer,wLineNum, lpwPatchNum);

Returns information regarding a specific input patch

that can be selected into an input

Parameters	HMIXER	hMixer

Identifies the mixer device to be used.

WORD  	wLineNum

The low byte indicates the mixer line to get the

connections information for.

The high byte indicates whether the line is an

input line or an output line. The following

macros (equates) are used for the high byte:

MIX_INPUT

MIX_OUTPUT

LPWORD	lpwPatchNum

Destination for the patch number.  A value of -1

signifies a user-defined patch.

Return Value	Returns zero if the function was successful.

Otherwise, it returns an error. Possible errors are:

MIXERR_INVALINPUT 	   illegal input line

MIXERR_INVALOUTPUT	   illegal output line

Comments	The range of wPatchNum must be from 0 to 1 less than

the number of software patches returned in

mixGetDevCaps.

See Also	mixSetPatch, mixGetPatchInfo

______________________________________________________________________

Syntax		mixSetPatch(hMixer,wLine, wPatchNum,lpPatch,wSize);

Allows the user to set the patch of an input to

another device.

Parameters	HMIXER	hMixer

Identifies the mixer device to be used.

WORD  	wLine

The low byte indicates the mixer line to get the

connections information for.

The high byte indicates whether the line is an

input line or an output line. The following

macros (equates) are used for the high byte:

MIX_INPUT

MIX_OUTPUT

WORD	wPatchNum

Specifies the patch number to set.  Each mixer

driver has a number of internal patches that

are selected by this parameter.

LPPATCHINFO lpInfo

If this parameter is not NULL, the wPatchNum

parameter is ignored and the PATCHINFO structure

pointed to is used to set the patch information.

The patch number assigned will be -1;

WORD    wSize

Specifies size of PATCHINFO structure

Return Value	Returns zero if the function was successful.

Otherwise, it returns an error. Possible errors are:

MIXERR_INVALINPUT 	   illegal input line

MIXERR_INVALOUTPUT	   illegal output line

MIXERR_PATCHMISMATCH   patch-to-line mismatch

Comments	The MIX_USER_CONNECTED bit may be OR'd with the patch

type to indicate a patch that is to be connected by

the user rather than one that is selected via

software control.  Applications should check this bit

at initialization time to advise users to make the

external connection.  If the patch number of a user-

connected patch is illegal, the driver's default

patch for that line will be used.

See Also	mixGetPatchInfo, mixGetPatch

______________________________________________________________________

Syntax		mixGetPatchInfo(hMixer,wPatchNum,lpInfo,wSize);

Returns information about a pre-defined patch.

Parameters	HMIXER	hMixer

Identifies the mixer device to be used.

WORD	wPatchNum

Specifies the patch number to set.  Each mixer

driver has a number of internal patches that

are selected by this parameter.

LPPATCHINFO lpInfo

If this parameter is not NULL, the wPatchNum

parameter is ignored and the PATCHINFO structure

pointed to is used to set the patch information.

WORD    wSize

Specifies size of PATCHINFO structure

Return Value	Returns zero if the function was successful.

Otherwise, it returns an error. Possible errors are:

MIXERR_INVALPATCH 	   patch number out of range

Comments	The current patch information for a line is

available by calling mixGetLineInfo.  Now that

patch information is stored in WIN.INI, the default

patch information is automatically overridden.  In

most cases the wPatchNum will be -1.  User defined

patch information is not returned by this call.

See Also	mixGetPatch, mixSetPatch, mixGetLineInfo

______________________________________________________________________

// DEVICE ORIENTED CALLS

mixGetDeviceName	- converts device type to device name

mixGetDeviceLines	- finds which lines have a specified device

mixSetDeviceConnections	- performs abstract device connection

mixGetDeviceConnections - determines device types connected

______________________________________________________________________

Syntax		mixGetDeviceName(dwDeviceType, lpDeviceName, wSize);

Returns information regarding a specific input patch

that can be selected into an input

Parameters	DWORD	dwDeviceType

A 32-bit value indication the device type

LPSTR	lpDeviceName

points to the destination for the device name

WORD  	wSize

buffers size pointed to by lpDeviceName

If wSize <= MIX_DEVICESHORTNAME, the three letter

standard device mnemonic string will be copied to

the buffer.  In all cases wSize will be the limit

of characters copied.

Return Value	Returns zero if the function was successful.

Comments	The device short name is intended for display in

dialog boxes and in win.ini's mixer configuration

settings.

See Also	mixGetDeviceLines, mixGetDeviceConnections

______________________________________________________________________

Syntax		mixGetDeviceLines(hMixer,lpDeviceLines);

///  Given a device type, this function will report the lines that the given

///  device is connected to.  If the association is not NULL, only

///  lines with the same association will be reported.  Otherwise, all

///  devices of the given type are reported on.

///

Parameters	HMIXER	hMixer

Identifies the mixer device to be used.

LPDEVICELINES lpDeviceLines

far pointer to DEVICELINES data structure

struct {

DWORD 	dwDeviceType;	 // aka technology

WORD	wNumDevices;	 // return value: # lines with device found

DWORD	dwLines;	 // return value: line map

DWORD	dwAssociation;	 // for exclusive search

} DEVICELINES;

typedef DEVICELINES FAR *LPDEVICELINES;

Return Value	Returns zero if the function was successful.

Comments	Be sure that the dwAssociation element of the

DEVICELINES structure is NULL unless you intend

to find a specific device line of which the

association has been established.

See Also	mixGetDeviceConnections, mixGetPatchInfo

______________________________________________________________________

Syntax		mixGetDeviceConnections(hMixer,lpDeviceConnect);

Given a device type, this function will return device

types that the given device type is connected to.

Input device types will yield reporting of output device

types connected and vice versa.  If the associationType

is given, only devices with the same association

will be reported.  Otherwise, all devices of the given

type are reported on.

Parameters	HMIXER	hMixer

Identifies the mixer device to be used.

LPDEVICECONNECT	lpDeviceConnect

long pointer to DEVICECONNECT structure

typedef struct{

DWORD dwInputDeviceType;

DWORD dwOutputDeviceType;

DWORD dwInputAssociation;

DWORD dwOutputAssociation;

} DEVICECONNECT;

Return Value	Returns zero if the function was successful.

Otherwise, it returns an error. Possible errors are:

MIXERR_INVALSTRUCTPTR  null lpDeviceConnect

Comments	An output device type is one that can be connected to

a mixer output and is distingquished by having the

MIX_LISTENER bit set in its dwDeviceType field.

See Also	mixSetDeviceConnections

______________________________________________________________________

Syntax		mixSetDeviceConnections(hMixer,lpDeviceConnect)

Given two device types, input and output, this function

will attempt to connect the two types.  If the

associationType and associationValue fields are not NULL,

only devices with the same association will be

connected.

Parameters	HMIXER	hMixer

Identifies the mixer device to be used.

LPDEVICECONNECT	lpDeviceConnect

long pointer to DEVICECONNECT structure

typedef struct{

DWORD dwInputDeviceType;

DWORD dwOutputDeviceType;

DWORD dwInputAssociation;

DWORD dwOutputAssociation;

} DEVICECONNECT;

Return Value	Returns zero if the function was successful.

Otherwise, it returns an error. Possible errors are:

MIXERR_INVALSTRUCTPTR  null lpDeviceConnect

Comments

See Also	mixGetDeviceConnections

______________________________________________________________________

CONTROL SETTING NOTES

The functions mixGetControl and mixSetInputControl

have a dwControl parameter and a dwSetting parameter.

Following the example set by Microsoft's volume settings, each

control setting will have associated with it a double word value

for specifying its setting.

dwControl		 description			format

---------		 -----------			------

MIX_SUPPORT_VOLUME	 volume control           	LLLL:RRRR scalar

MIX_SUPPORT_LRVOLUME       left-right volume control	LLLL:RRRR scalar

MIX_SUPPORT_ALC		 Auto Level Control		LLLL:RRRR on/off

MIX_SUPPORT_BMT		 B-M-T equalization		--BB:MMTT scalar

MIX_SUPPORT_CROSSOVER	 crossover change     		MIXCROSSCAPS

MIX_SUPPORT_LOUDNESS	 loudness equalization		LLLL:RRRR on/off

MIX_SUPPORT_MUTE		 mute - don't change volume	LLLL:RRRR on/off

MIX_SUPPORT_REVERB	 reverb 			LLLL:RRRR scalar

MIX_SUPPORT_STEREOENHANCE  stereo enhance 		LLLL:RRRR on/off

MIX_SUPPORT_CUSTOM1	 custom effect #1		LLLL:RRRR on/off

MIX_SUPPORT_CUSTOM2	 custom effect #2		LLLL:RRRR scalar

Scalars are unsigned values from 0-65535 except in the case of BMT

where they are unsigned values from 0-255.  These values should be

interpolated to match the mixer hardware's scale.

On/Off values are ON for any non-zero value.

//////////////////////////////////////////////////////////////////////

MCI INTERFACE

Media Vision is now providing an MCI driver for controlling the mixer

hardware.  For many of you, this will make the control of mixing

functions much easier than it ever has been.

MCI, as you must know, stands for Media Control Interface.  In the

case of a mixer, there is no media (medium).  When we speak of

CDs, wave files, MIDI, we connote some sort of media transport,

(ie. position within the data, support for PLAY, STOP, REWIND, etc)

A PLAY command sent to a mixer does not make sense.  In fact most MCI

commands have little meaning for a mixer.

Still, MCI is powerful.  One of its outstanding features is its

ability to convert strings into driver commands.  We have, therefore,

developed an MCI driver for Multimedia Windows.

The documentation for the MCI mixer is found in the file

mcimixer.doc.

//////////////////////////////////////////////////////////////////////

MIXER NOTES:

The mixSetState function allows timed fades.  If it would be useful

to developers we can add the capability for timed fades to the

mixSetControl function.  Let us know if you want this.  Soon.

User-defined patches are not accessible through mixGetPatchInfo.

The mechanism of user-defined patches may change in the future.

//////////////////////////////////////////////////////////////////////

GLOSSARY:

line - a stereo input or output of a mixer

patch - the association of a line to a particular sound producing

or recording device

connection - this term refers to a mixer's internal input-to-output

routing of an audio signal

control	- the capability to modify an audio signal (ie volume)

cross-fade  - fading one or more controls up while fading other out

//////////////////////////////////////////////////////////////////////