From: ftg0673@tamsun.tamu.edu (Rick Grevelle)

Subject: HP48 HACKIT Library w/ Bank Shifting

Lines: 557

This is an enhanced version of the HACKIT thing.  It includes several new

features which further facilitate hacking in the 48, and are as follows:

o   Multifunctional PEEK will now allow the normally hidden

32k of ROM to be peeked.  When flag 14 is clear the 32k

of memory between #70000h and #7FFFFh is RAM.  When the

flag is set this is shifted to #F0000h through #FFFFFh,

and #70000h through #7FFFFh becomes what was previously

covered by RAM; this configuration is identical to that

of the built in memory scanner.

o   Bank switching capabilities have also been incorporated

in the SEEK scheme, functioning exactly as described in

the PEEK routine.

o   DIR-> is a new addition to the OUT-> command; it allows

a directory in level 1 to be decomposed on the stack in

to its constituent parts.  Due to this change, it's not

necessary to include the RCLDR scheme in the library as

it was added to the original HACKIT post only after all

first attempts to produce a successful DIR-> had failed.

o   Flags -28 and -29 have now been combined into one flag,

15.  This flag affects the way in which the ->ASCI, and

ASCI-> routines handle code objects.  Currently a ->STR,

and STR-> scheme is being conceived that will provide a

superior editing scheme for queer objects.

At the risk of being reiterative, I'm including an improved version of the

documentation which appeared in the original posting of this library.  It

should be a bit more articulate, and contain examples for those whose time

is allowing.  Much work still remains to be done regarding the completely

bullet proof hacking library, but since all the fun seems to be in getting

there, hopefully it will take a long time.

A final word of caution; do *not* attempt to implement a bank shift using

either PEEK, or SEEK, if this library is stored in port 0, or anywhere in

the address space being displaced.  This means both of the 32k banks that

are affected by setting flag 14 are off limits, as far as this feature is

concerned, for the storing of this library.  Should there be any question

about this, or anything else, feel free to call, or write, and I shall be

glad to provide any answers of which I might be capable :-).

Rick Grevelle

(409) 774-1169

ftg0673@tamsun.tamu.edu

HACKIT

==============================================================================

HACKIT is a library of utilities designed to facilitate hacking in the HP48SX.

The fourteen programs it contains were conceived for two fundamental purposes:

o   Exploration, documentation, and accessing of the machine code, and RPL

operating system contained in the 48's vast ROM.

o   Construction, and manipulation of the various object types not supported

by the 48's editor, user language commands, functions, or operations.

->ASCI

This is a generalized binary-to-ascii conversion scheme that returns a string

of hexadecimal characters which are the equivalent internal representation of

the argument.  When USER FLAG 15 is set both the prolog and the length of code

objects are truncated meaning that the result string contains in-line machine

code only.  When this flag is clear, which is the default, truncation does not

occur.  All other object types are unaffected by the flag setting.

ASCI->

Reversing the result of the previous routine is made possible with this

scheme,

which converts ascii-to-binary, taking as its argument a string of hexadecimal

characters. When USER FLAG 15 is set all strings are treated as in-line

machine

code, whereby a code object would be the result.  When flag 15 is clear one of

two things happens depending on the string argument itself, and is explained

in

the following summary.

IMPORTANT POINTS

o   The above routines were intended to provide a means for *toggling* stack

objects back between their internal string representation and immediately

executable form, which means ROM objects must be dealt with differently.

An important attribute associated with these two schemes is their ability

to differentiate between ROM and RAM objects.

o   When an argument taken by the ->ASCI routine is a ROM object, a character

string representing the five nibble address where that object is located

in ROM is returned.  Strings containing only five characters that are

used

as arguments for ASCI-> are treated as pointers; solitary prologs are not

permitted though (i.e. "D9D20", "C2A20", "11920").  Future versions

should

be more than capable of dealing with such complexities as this.

o   Spaces and newline carriage returns are allowed in strings that are to be

converted by ASCI->.  This helps to alleviate some of the confusion when

looking at long strings of hexadecimal characters in that spaces can now

be used to separate groups of characters, and newline carriage returns to

segment the string into several lines when editing.

o   Any object encoded using ->ASC can be unencoded using ASCI->.  There's no

longer a need for these silly schemes to delete newline carriage returns,

or calculate a new checksum to tack onto an ->ASC encoded object that has

been altered in order to get ASC-> to work.  ->ASC, and ASC-> were

written

to provide a safe means of transferring data, and not for hacking.  It is

therefore not recommended to use these two schemes in place of ->ASC, and

ACS->.

o   ASCI-> performs equally well on strings of odd number lengths because the

RPL segments of the program correctly utilize the block allocate routine

at #61C1Ch.  But because of its very nature, it's still quite possible to

suffer a memory loss from abusing this routine; so be careful.

Example:  Construct the DUP command from it ASCII string representation.

_________________________               _________________________

|{HOME}                 |               |{HOME}                 |

|-----------------------|               |-----------------------|

|4:                     |               |4:                     |

|3:                     |               :3:                     |

|2:                     |               |2:                     |

|1:              "78BF1"|               |1:                  DUP|

|### ### ### ### ### ###|               |### ### ### ### ### ###|

-------------------------               -------------------------

1) Enter the string "78DF1".            2) Implement ASCI-> to obtain

the command DUP.

SCRC

Calculates the checksum of a string of hexadecimal characters as if they were

in immediately executable form.  So objects which have been converted to their

internal hexadecimal character string form (ASCII) using ->ASCI will have the

same checksum when SCRC is used on the ASCII string, as they would have had in

their binary form using the BYTES command.  This allows queer objects such as

libraries and backups that contain checksums which do not include the complete

data structure to be altered, or even constructed from scratch, in the 48.  So

for an example a trivial backup object will be built utilizing SCRC, ->ASCI,

and the ASCI-> routines.

Example:  Build a backup object named "Hex" containing the hexadecimal

integer, # 123456789ABCDEF0h.

o  Construct the backup object's appropriate name.

_________________________               _________________________

|{HOME}                 |               |{HOME}                 |

|-----------------------|               |-----------------------|

|4:                     |               |4:                     |

|3:                     |               |3:                     |

|2:                     |               |2:                     |

|1:                'Hex'|               |1:      "84E2030845687"|

|### ### ### ### ### ###|               |### ### ### ### ### ###|

-------------------------               -------------------------

1) Enter the name 'Hex'.                2) Implement ->ASCI on the

name.

_________________________               _________________________

|{HOME}                 |               |{HOME}                 |

|-----------------------|               |-----------------------|

|4:                     |               |4:                     |

|3:                     |               |3:                     |

|2:                     |               |2:                     |

|1:           "30845687"|               |1:         "3084568730"|

|### ### ### ### ### ###|               |### ### ### ### ### ###|

-------------------------               -------------------------

3) Edit the string to delete            4) Concatenate "30" by using

the first five characters.              the plus (+) key.

o  Prepare the backup object's contents for concatenation.

_________________________               _________________________

|{HOME}                 |               |{HOME}                 |

|-----------------------|               |-----------------------|

|4:                     |               |4:                     |

|3:                     |               |3:                     |

|2:         "3084568730"|               |2:         "3084568730"|

|1:  # 123456789ABCDEF0h|               |1: "E4A20510000FEDCBA..|

|### ### ### ### ### ###|               |### ### ### ### ### ###|

-------------------------               -------------------------

1) Enter the integer data.              2) Implement ->ASCI once again.

o  Make final preparations for checksumming.

_________________________               _________________________

|{HOME}                 |               |{HOME}                 |

|-----------------------|               |-----------------------|

|4:                     |               |4:                     |

|3:                     |               |3:                     |

|2:                     |               |2: "3084568730E3A2051..|

|1: "3084568730E4A2051..|               |1:              "33000"|

|### ### ### ### ### ###|               |### ### ### ### ### ###|

-------------------------               -------------------------

1) Concatenate the previous             2) DUP the result string, and

two result strings using                use SIZE to obtain its size.

the plus (+) key again.                 Add 15 to the size, convert

the result to a hexadecimal

number, and enter backwards

as a five character string.

_________________________               _________________________

|{HOME}                 |               |{HOME}                 |

|-----------------------|               |-----------------------|

|4:                     |               |4:                     |

|3:                     |               |3:                     |

|2:                     |               |2: "330003084568730E4..|

|1: "330003084568730E4..|               |1:             "119200"|

|### ### ### ### ### ###|               |### ### ### ### ### ###|

-------------------------               -------------------------

3) Perform a SWAP, and using            4) Append the string "119200"

the plus (+) key, prepend               to the end of the string,

the length onto the front               DUP the result, and using

of the string.                          SCRC calculate the object's

proper checksum.

o  Make final preparations for coverting the string.

_________________________               _________________________

|{HOME}                 |               |{HOME}                 |

|-----------------------|               |-----------------------|

|4:                     |               |4:                     |

|3:                     |               |3:                     |

|2: "330003084568730E4..|               |2:              "26B20"|

|1:               "0B41"|               |1: "330003084568730E4..|

|### ### ### ### ### ###|               |### ### ### ### ### ###|

-------------------------               -------------------------

1) Enter the four hexadecimal           2) Enter the prolog for the

characters of the checksum              backup object backwards,

backwards in a string, and              and perform another SWAP.

append this to the string

for which the checksum was

calculated.

_________________________               _________________________

|{HOME}                 |               |{HOME}                 |

|-----------------------|               |-----------------------|

|4:                     |               |4:                     |

|3:                     |               |3:                     |

|2:                     |               |2:                     |

|1: "26B20330003084568..|               |1:           Backup Hex|

|### ### ### ### ### ###|               |### ### ### ### ### ###|

-------------------------               -------------------------

3) Prepend the prolog using             4) Convert the string using

the plus (+) key one last               ASCI-> to its binary form.

time.

SEEK

Appearing first in his Processor Notes, SEEK is in fact a modified version of

Alonzo Gariepy's FIND routine.  SEEK was specifically modified to run in the

48; it's one third the size of the original version for the 28, and should run

a bit quicker too.

Alonzo's predominately register, rather than memory oriented algorithm remains

unchanged.  Recall the overall increase in speed was only about 10% over that

of the brute force approach, which he attributed to an increased time spent in

the looping structures.  Because it demonstrates several programming features

specific to the 48, I've taken the liberty of renaming the routine.

The way to use this program is to specify a memory pattern and a place to

start

looking.  A memory pattern can be sequence of up to fifteen nibbles, and a one

nibble length (one minus the number of nibbles in the pattern).  An

application

for SEEK could be to locate the occurrences of the instruction 808C in memory.

To do this enter # C8083h in level two, and a starting place such as # 0h.

All

that remains to be done is to implement SEEK, and to continue doing so to find

any subsequent instances.  This is because the result returned will be a

binary

integer which is the address one higher than where the pattern exist in

memory.

IMPORTANT POINTS

o   Bank shifting capabilities have been incorporated into SEEK, allowing the

normally hidden 32k bank of ROM to be searched.  With USER FLAG 14 clear,

the 32 kilobytes of memory between #70000h, and #7FFFFh, is RAM.  Setting

flag 14 shifts the RAM to #F0000h through #FFFFFh, and reconfigures what

was previously hidden ROM in its place.

o   This version does not stop until it finds an instance, or scans the

entire

address space.  So even though the routine is fast, due to the size of

the

48's address space, matching certain patterns could take several seconds.

Example:  Determine the first occurrence of the ARRAY prolog, # 029E8h,

in the normally hidden 32k bank of ROM. (flag 14 set)

_________________________               _________________________

|{HOME}                 |               |{HOME}                 |

|-----------------------|               |-----------------------|

|4:                     |               |4:                     |

|3:                     |               |3:                     |

|2:             # 29E84h|               |2:             # 29E84h|

|1:             # 70000h|               |1:             # 72001h|

|### ### ### ### ### ###|               |### ### ### ### ### ###|

-------------------------               -------------------------

1) Enter the bit pattern with           2) The result returned is the

the number of nibbles minus             one higher than where the

one, and the address where              actual pattern was found.

to begin searching.

PEEK

Speed is still an essential attribute of this dual functioning PEEK.  Only the

variable length version will slow slightly when large result strings hundreds

of characters long are returned.  The routine is smart enough to know which of

the PEEKs to implement based on the arguments it's given.  When string results

are preferred, level two should contain a binary integer that's the address to

be peeked, while level one should be a real number representing the length of

the result string.  The bank shifting features as described in SEEK scheme are

also available in this version of PEEK; an example follows.

Example:  Using PEEK and ASCI->, recover the message array from the

hidden ROM found in the previous example. (flag 14 set)

_________________________               _________________________

|{HOME}                 |               |{HOME}                 |

|-----------------------|               |-----------------------|

|4:                     |               |4:                     |

|3:             # 29E84h|               |3:                     |

|2:             # 72000h|               |2:             # 29E84h|

|1:                  641|               |1:      Array of String|

|### ### ### ### ### ###|               |### ### ### ### ### ###|

-------------------------               -------------------------

1) Subtract one from the above          2) Now implement ASCI-> on the

result, enter the number of             result to obtain the ARRAY.

nibbles to be returned, and

implement PEEK.

POKE

Complimenting PEEK is this dual functioning variable length POKE scheme which

will accept either a string of hexadecimal characters, or a binary integer for

the level two argument that represents the data to be poked.  Level one should

be a binary integer that is the address to poke.  It is important to note that

when the level two argument is a binary integer, the wordsize of that specific

integer will be used to determine the number of nibbles to poke, and isn't in

any way dependent upon the current wordsize returned by RCWS.

OUT->

Multifunctioning OUT-> encompasses several of the object types on which OBJ->

will not work.  In addition, and analogous to the LIST-> command, ALG-> and

PRG-> are intended to function on algebraics, and programs.  ARR-> functions

identically to ARRY->, but works on all the various array types such as the

array of string, algebraic, list, etc.  DIR-> decomposes a directory that is

in level one into its constituent parts.  XLIB-> has also been included; it

decomposes visible, and hidden XLIBs into their library and command numbers.

Example:  Decompose the Array of String obtained in previous example

into its constituent parts.

_________________________               _________________________

|{HOME}                 |               |{HOME}                 |

|-----------------------|               |-----------------------|

|4:                     |               |4: "Try To Recover Me..|

|3:                     |               |3: "Replace RAM, Pres..|

|2:             # 29E84h|               |2: "No Mem To Config ..|

|1:      Array of String|               |1:               { 16 }|

|### ### ### ### ### ###|               |### ### ### ### ### ###|

-------------------------               -------------------------

1) Using the array obtained             2) Recompose the array again

in the previous example,                by using the ->ARR routine.

implementing OUT-> will                 The result will look like

reveal the strings.                     that of frame 1).

->ALG

Be careful with this one; the stack arguments must be in strict RPN order.  As

before, ->ALG is analogous to ->LIST, only the result is an algebraic, rather

than a list.

->ARR

This is my version of ->ARRY; it will build an array of any kind providing all

stack arguments are of the same type. (i.e. strings, reals, algebraics, etc.)

It functions identically to ->ARRY, except that real and complex numbers can't

be mixed.  Level one can be either a real number, a list containing a real, or

a list containing two reals that is the size the resulting array is to be.

->DIR

Builds a directory from the stack on the stack.  Level one must contain a real

which is the total number of variables that the directory is to contain, level

two a global name, and level three the contents of that global name.  Because

the entire process utilizes dynamic RAM, it is relatively fast for moderately

sized directories, but slows substantially on very large ones.  Using both the

DIR-> feature found in the OUT-> command along with this routine, a directory

on the stack can be effectively toggled between its constituent parts, and its

composite form.

->PRG

Functions identically to ->LIST, only the result is a program, and not a list.

->XLIB

Builds an XLIB from two stack arguments.  Level two can be either a real

number

or a binary integer that's the number of the library to which the XLIB

belongs.

Level one must be the same argument type as level two and should be the

command number of the desired XLIB.

ADDR

Returns the address where the following stack arguments are located in memory:

any ROM object, any XLIB belonging to a library stored in a port, any TAGGED

that's either a BACKUP object or a LIBRARY stored a port.  One final attribute

associated with ADDR that merits mentioning, regards those XLIBs that belong

to the three intrinsic ROM libraries with a link table.  This routine can just

as easily locate these address, as it can those in port memory.  Even if the

memory location happens to be that of the hidden ROM, ADDR will determine its

address.

RCLIB

This multifunctional routine is to a library what RCL is to a directory.

RCLIB

will recall a library as a fully operational directory.  All that is necessary

to do is to put either the library's name or (real) number in level one.

Again

this is slowed greatly on very large libraries.  RCLIB will work on any

library

including ROM (i.e. 2, 240, 1792).  Finally, the contents of any library's

XLIB

can be recalled to the stack; simply put the appropriate XLIB in level one,

and

the result will be comparable to using RCL on variable stored in a directory.