Profile Calculator is a 32-bit program designed for Windows 95 and NT.

Overview: INF Files

A complete reference to the structure and content of a Windows 95 INF file is included in the publication "Device Information Files." Additional detailed information on modem INF file structure is contained in the Microsoft MDK.DOC document. An overview of INF files is also discussed in the  Microsoft Windows 95 Resource Kit appendix. INF files are typically located in the INF folder of your Windows folder.

Overview: Profile Calculator

Use Profile calculator to create, edit or translate properties lines within Windows 95 modem INF files. Modem INF properties lines contain device properties settings that are transferred from an INF file to the Windows 95 Registry at the time a device is installed. Profile calculator is bi-directional: it can translate text values into an INF properties line or translate existing properties lines into text.

The following are examples of typical properties lines found in a modem INF:

HKR,, VoiceProfile,    1, 27,03,00,00

HKR,, Properties, 1, 80,01,00,00, ff,00,00,00, 00,00,00,00, 07,00,00,00, 07,00,00,00, 77,03,00,00, 00,c2,01,00, 40,38,00,00

Notes

	A properties line is a single line in an INF file without text wrapping.

	When Profile Calculator first starts, a list of values appear in text, visible in the large lower window. The calculated INF properties line is visible in the long upper window. Double-clicking on specific values in the lower window toggles the value on or off while reflecting the changes to the properties line in the long upper window.

Overview: Profile Configuration Files (.PRF)

Profile Calculator uses several .PRF files to configure itself for the appropriate style structure:

	VMODEM.PRF provides a Voice Profile template. For example:

HKR,, VoiceProfile,    1, 27,03,00,00

	DMODEM.PRF provides a Data Properties structure template. For example:

HKR,, Properties, 1, 80,01,00,00, ff,00,00,00, 00,00,00,00, 07,00,00,00, 07,00,00,00, 77,03,00,00, 00,c2,01,00, 40,38,00,00

Notes

	Properties lines are usually a single line in an INF without text wrapping.

	You can edit an .PRF file because it is in the ASCII text format. Remarks about its structure and comments are provided in each .PRF file.

Overview: Converting Settings into an INF Profile

Profile Calculator can construct or edit either Voice Profile or Data Properties lines. When you start Profile Calculator, the default Data Properties line is presented with a list of values in text, visible in the large lower window, and the calculated properties line, visible in the long upper window.

Double-clicking on specific values in the lower window toggles the value on or off while reflecting the changes to the properties line in the long upper window. Settings requiring a decimal value are entered in the small upper window or selected from a drop down list containing standard values for that setting. After you define all values, you can use the copy and paste to transfer the finished properties line from the long upper window of the Profile Calculator into the clipboard.

Notes

	Settings requiring a decimal value are accompanied by an optional comments box explaining what the setting is for.

To convert an INF properties line into text

You can use Profile Calculator to translate properties line into text.

1	Copy a properties line from an INF file.

2	Switch to Profile Calculator.

3	Click the Paste button.

A translation of the property values appears in the lower window.

To switch between different templates

1	On the File menu, click Open.

2	Click VMODEM.PRF. This reconfigures Profile calculator for that type of property structure.

Currently there are two template files: DMODEM.PRF and VMODEM.PRF

Menu commands in Profile

File Menu

Open: opens a .PRF file that changes the type of structured properties line you want to edit.

Exit: Closes Profile Calculator. This does not save any current calculations or values.

Edit menu

Cut: Cuts marked text to the clipboard buffer.

Copy: Copies marked text to the clipboard buffer.

Paste: Pastes text ONLY into the long upper window.

Clear: Clears or erases all values in the long upper window.

View Menu

Standard: The normal default which provides an "Intel, byte reversed display of the properties line". This line can normally be pasted directly into the INF without further adjustment.

Hexadecimal: Displays the properties line without the Intel byte reversal.

Custom: Displays the properties line using formatting exactly as specified in the .PRF file with the Intel byte reversal.

Show Comments: Toggles the Comments box option on and off.

Command buttons in Profile

	Copy: Copies the contents of long upper window to the clipboard.

	Paste: Pastes the contents of the clipboard to the long upper window and updates the lower window.

	Clear: Resets all values in the lower window to OFF and then updates the long upper window.

	Update: Forces an update of the lower window with the contents of either of the upper windows. This is useful after you manually edit the contents of the upper windows.

Tabs in Profile

There may be one or more numbered tabs along the top of the lower window. These tabs provide additional pages or views that continue the structure. Click the page that presents the portion of the structure you want to edit.

Known Problems:

Closing the Comments dialog via the close button will cause the dialog to display behind the main application window until the application is restarted.